# BEARING HOUSINGS

Phase 2 | Quick Counter Cheat Sheet

| 👁 WHAT IT IS • Housing unit supporting a bearing • Provides alignment and mounting • Often pre-assembled 👁 COMMON TYPES • Pillow block • Plummer block • Flanged housing • Take-up units 📏 KEY MEASUREMENTS • Shaft diameter • Mounting bolt pattern • Housing type • Insert bearing type 📏 OPERATING LIMITS • Alignment tolerance limited • Speed depends on insert • Environment dependent | 👁 10-SECOND VISUAL CHECK • Grease nipple present • Set screw or eccentric lock • Mounting holes visible ⚠ COMMON MISTAKES • Wrong shaft size • Incorrect locking method • Ignoring environment sealing ⚠ WHEN TO ESCALATE • Severe misalignment • Washdown environments • High-speed shafts 👁 GOOD / NOT IDEAL • Good: conveyors, fans • Not ideal: high-precision alignment • → Consider plummer blocks |
| --- | --- |

FINAL CHECK:  SHAFT ✓   HOUSING ✓   LOCK ✓   SEAL ✓   LUBE ✓