# UNIVERSAL JOINTS

Phase 2 | Quick Counter Cheat Sheet

| 👁 WHAT IT IS • Transmits torque through an angle • Allows angular misalignment between shafts • Common in driveline and power transmission 👁 COMMON TYPES • Single Cardan (standard U-joint) • Double Cardan (CV-style) • Heavy-duty / greaseable • Sealed / non-greaseable 📏 KEY MEASUREMENTS (CRITICAL) • Cap diameter • Overall span (cap to cap) • Snap ring location (inside vs outside) • Spline size or count (if applicable) 📏 OPERATING LIMITS • Best at small to moderate angles • Speed fluctuation increases with angle • Not constant velocity (single joint) | 👁 10-SECOND VISUAL CHECK • Cross-shaped spider • Four bearing caps • Circlips or snap rings retain caps • Yokes at 90° to each other ⚠ COMMON MISTAKES • Measuring only one cap • Ignoring snap ring style • Mixing greaseable and sealed types • Using excessive operating angle ⚠ WHEN TO ESCALATE • High-speed driveline vibration • Repeated joint failure • Large operating angles • Unknown shaft application 👁 GOOD / NOT IDEAL • Good: drivelines, PTO shafts, industrial machinery • Not ideal: high-speed precision or constant velocity • → Consider CV joints |
| --- | --- |

FINAL CHECK:  CAPS ✓   SPAN ✓   SNAP RINGS ✓   ANGLE ✓   SPEED ✓