# NEEDLE ROLLER BEARINGS

Phase 2 | Quick Counter Cheat Sheet

| 👁 WHAT IT IS • Bearing using long, thin rollers • High load capacity for size • Compact radial design 👁 COMMON TYPES • Caged needle rollers • Drawn cup • Full complement • Inner ring or shaft-running 📏 KEY MEASUREMENTS • Bore (shaft size) • Outer diameter • Width • Roller length 📏 OPERATING LIMITS • Limited axial load capacity • Requires hardened race • Sensitive to misalignment | 👁 10-SECOND VISUAL CHECK • Very thin cylindrical rollers • Minimal cage clearance • Compact outer ring ⚠ COMMON MISTAKES • Running on unhardened shaft • Ignoring lubrication needs • Misalignment in housing ⚠ WHEN TO ESCALATE • Unknown shaft hardness • Shock or oscillating loads • High-speed applications 👁 GOOD / NOT IDEAL • Good: gearboxes, transmissions • Not ideal: axial loads • → Consider ball bearings |
| --- | --- |

FINAL CHECK:  BORE ✓   OD ✓   WIDTH ✓   HARDNESS ✓   LUBE ✓